# TA failures

Use the sentence in the window or in data/notify.log. Do not restart the oven to clear a board-only failure.

| What you see | What to do |
| --- | --- |
| check says FAIL and names the oven port | Find the COM port again. Set oven_port. Do not start the oven. |
| check says a board did not answer SSH | Ping that address. Fix the PC route or the board static address. See the setup manual. |
| missing rofc_11, rofc_21, or rofc_51 | Copy those notebooks onto that board. The repo copies are not used. |
| an oven run is already active | Do not start a second one. Use the open window, or clear a stale OVEN_OWNED only if no window is open. |
| BOARD_BUSY or OVEN_OWNED on board_02 | Group B must wait, or you clear a stale file only if that window is closed. |
| 2/3 csv and a notebook name | Read that board log under data/boards. The other group keeps running. |
| plot window is empty | The oven window must already be recording. Leave it open. |
| mail not sent: in data/notify.log | Fix secrets/notify.json. The run itself is still valid. |
| oven crash mail to both groups | Expected. The shared temperature stopped. Restart only after the oven is safe. |