# TA week

One oven window for the whole stretch. Group B joins later on the other board. Do not average or delete CSVs until both windows are closed and the raw folders have been copied.

## Before group A

- check_oven.bat says OK. The port and both board addresses were confirmed in the setup manual.

- oven_board_01.bat measures board_01 only. It must not be edited to add board_02.

- Leave the lab session logged in. Tell group A not to log off.

## When group B arrives

- The oven window is still open. Group B does not start it.

- check_board_02.bat says OK. board_02_only.bat measures board_02 and writes data/boards/board_02/groupB/.

- Group B uses the current oven temperature. A different profile means stopping the oven run. There is no second oven program.

## After both groups finish

- Ctrl+C in each window, or confirm the windows have already exited.

- Copy data/boards/board_01/ and data/boards/board_02/groupB/ somewhere safe.

- Only then may average_outputs.bat be used, and only after typing YES. It deletes raw CSVs under pynq_run_data and does not read the groupB folder.

If a window was closed without Ctrl+C and no matching window is open, delete the stale OVEN_OWNED or BOARD_BUSY file before the next start. Do not delete it while a window is running.